Your backbone choice (Hybrid CNN+Transformer) is validated, not just a reasonable guess

A 2024 paper, "1D-CNN-Transformer for Radar Emitter Identification and Implemented on FPGA", uses almost exactly your architecture: 1D-CNN extracts local pulse features (rising/falling edges, envelope shape), Transformer captures long-range dependencies across the sequence, combined into a lightweight hybrid — specifically built for edge deployment. That's DADS's HybridBackbone design, independently arrived at by published research for the same reason (edge-deployable, not just "big transformer for accuracy"). Keep it.

Your open-set approach (metric head + distance threshold) matches the current consensus — with one clear next step

The dominant 2025 pattern across this literature is metric-learning-based open-set recognition: train an embedding space (triplet loss, prototypical networks) where distance-to-nearest-known-class becomes the "is this novel?" signal, rather than trusting softmax confidence. That's exactly DADS's dual-head design (classifier + L2-normalized metric head, OOD via nearest-prototype distance in Qdrant).

The frontier has moved one step past plain thresholding, though: adversarial reciprocal point learning (ARPL) (2025) explicitly trains against synthetic "reciprocal points" to actively push open space away from known-class regions, rather than relying on a threshold tuned after the fact. This is a legitimate, citable upgrade path — not a sign you did something wrong, but a concrete "next chapter" if you want the open-set method itself to be a research contribution rather than an engineering choice.

Your deinterleaving approach (DBSCAN clustering) is where the literature has moved on since you built it

This is the one honest gap worth flagging directly. DADS's deinterleaver.py uses classical density-based clustering (DBSCAN) over frequency/PW/AOA. A March 2025 paper, "Radar Pulse Deinterleaving with Transformer Based Deep Metric Learning", trains a transformer with triplet loss specifically for this task and reports an adjusted mutual information score of 0.882 on synthetic data — a learned embedding replacing hand-picked clustering features and distance metrics. If you want DADS to reflect current state-of-the-art rather than a solid classical baseline, this is the most concrete, directly-comparable upgrade: swap or ablate DBSCAN against a learned deinterleaving embedding, trained the same way your classifier already is.

What the defense-industry side confirms, without specifics I can verify

Public commentary from EW vendors (L3Harris, CRFS) confirms the direction: AI-enhanced ESM for real-time emitter pattern recognition and behavior prediction is an active, real industry trend — but none of that public material gets into specific model architectures or deployment numbers, because that detail is exactly what stays classified/ITAR-controlled. So DADS's overall premise (deep-learning-based real-time PDW classification, TensorRT for edge inference speed) is directionally aligned with where the field is actually moving — I just can't cite you a number from an actual fielded system, because that's not published, and shouldn't be.

My honest recommendation

- Keep the hybrid CNN+Transformer backbone — it's independently validated by recent published work for this exact problem shape.

- Keep the metric-learning open-set design, but consider adding an ARPL-style loss as a comparison arm — this turns "we picked a threshold" into "we compared threshold-based vs. adversarially-trained open space rejection," which is a stronger PhD contribution.

- Treat the deinterleaving module as your weakest link relative to 2025 literature — DBSCAN works and is defensible as a baseline, but a transformer-based metric-learning deinterleaver (mirroring the March 2025 paper) is the most concrete, publishable upgrade available to you right now, and it reuses infrastructure (triplet loss, embeddings) you've already built for the classifier.

- Don't chase "military-grade" as a framing — the honest, defensible framing for a PhD is "aligned with current unclassified open-set/deinterleaving literature," which is exactly what you have grounds to claim.

Sources:

- 1D-CNN-Transformer for Radar Emitter Identification and Implemented on FPGA

- Open set recognition of radar specific emitter based on adversarial reciprocal point learning

- Radar Pulse Deinterleaving with Transformer Based Deep Metric Learning (arXiv 2503.13476)

- Few-Shot Learning for Radar Emitter Signal Recognition Based on Improved Prototypical Network

- Self-supervised and semi-supervised learning for few-shot specific emitter identification using CNN-transformer

- A framework for radar signal deinterleaving and parameter estimation based on split pulse features

- What is Electronic Warfare? — L3Harris

- Sources: Attention-Based Radar PRI Modulation Recognition · MODSIM World 2025 — multifunction radar simulation · RadSimReal (CVPR 2024) · The Turing Synthetic Radar Dataset · RadSeg synthetic radar dataset (ICASSP 2024)

- Sources: User interface design and evaluation of a shipboard electronic warfare console · Dashboard Design for Real-Time Situation Awareness (ISA-101) · From Data To Decisions: UX Strategies For Real-Time Dashboards